Satellite altimetry in the Czech Republic: status 2007
J. Klokočník 1,  J. Kostelecký 2,3

1) CEDR - Astronomical Institute, Academy of Sciences of the Czech Republic,

CZ-251 65  Ondřejov Observatory, Czech Republic, jklokocn@asu.cas.cz

2) CEDR - Research Institute for Geodesy, Topography and Cartography,

CZ-250 66 Zdiby 98, Czech Republic, kost@fsv.cvut.cz

 3)  Department of Advanced Geodesy, Czech Technical University,

CZ- 166 29 Praha 6, Thákurova 7, Czech Republic, kost@fsv.cvut.cz

ABSTRACT
    The aim of this presentation is to inform about works in satellite altimetry in the Czech republic during the last 15 years and to provide an outline of the results. The results come from two groups, one is from CEDR (Center for Earth Dynamics Research) and the second is from Military Geographic and Hydrometeorologic Office. We also cooperated with NOAA and NIMA. (i) We worked out  - and used repeatedly - a method based on long-term averaged crossover altimetry data (from missions ERS 1, 2, Envisat and Geosat). With these (generated from orbits using tested geopotential models) we computed latitude lumped coefficient (LLC) discrepancies, compared them to the LLC errors projected from the covariance matrices of these models and from the comparison assessed the  accuracy of the gravity models. (ii) We used single and dual satellite crossover data (SSC and DSC) to refine gravity field models. (iii) We also utilized the DSC between low and higher altimetry orbits to detect possible offsets among the terrestrial reference frames of the altimetry missions involved. (iv) The potential of geoid W0 has been derived by means of combining altimetry, levelling, GPS height and gravity data, and in turn, an offset between different world height systems has been estimated. (v) We also studied the theory of bistatic satellite altimetry, namely the distribution of reflecting points and the accuracy of their determined surface heights, and most recently the determination of  the specular reflecting points on the Earth represented not only by a sphere but also an  oblate ellipsoid.  

Key words:    satellite altimetry, accuracy of the Earth gravity field models, 
single- and dual- satellite crossovers,  world height systems, bistatic satellite altimetry

1.  ACCURACY ASSESSMENT OF GRAVITY FIELD MODELS BY MEANS OF   

    CROSSOVER ALTIMETRY DATA
     It is one thing is to compute an Earth gravity field model – a set of harmonic geopotential coefficients Clm, Slm (also known as Stokes parameters), of degree l and order m. Just as demanding is to estimate their actual accuracy (not only internal precision from the covariance matrix); this becomes more and more difficult as their apparent errors decrease (Klokočník et al, 2004; Novák et al, 2006). Various methods are in use (e.g., Lerch 1991,  Lemoine et al,  1998). We have developed and used a method based on crossover satellite altimetry data, very often independent of the global solutions and sufficiently accurate, at least in the pre-GRACE era.
     The radial direction is crucial for altimetry. The formulae for the radial, static gravity field induced error as a time series, are well known (e.g. Colombo 1984, Wagner 1989). But the time series of error does not provide any useful insight into its source on the orbit or in altimeter corrections because direct altimetry is dominated by the much larger uncertainty of the detailed geoid at the surface. That is revealed in differences of altimeter heights at crossovers (eliminating the influence of the geoid error) and (in comparison) displaying the geographical distribution of the radial error due to the geopotential in latitude and longitude. This last is accomplished using Rosborough's (1986) transformation (Rosborough and Tapley, 1987)  by projecting the variance-covariance matrices of the tested gravity models to yield expected orbit geopotential errors of SSCs (single-satellite crossovers) geo-graphically. But such information, while useful as an overview, and used many times, says nothing about the spectral quality of the geopotential errors. Following the natural formulation of spherical harmonics, this ‘Rosborough spectrum’ is most readily displayed by order in terms of so-called latitude lumped coefficients (LLC). They were defined (for SSC) in Klokočník and Kobrle (1992) and Klokočník et al. (1992), and applied in accuracy tests of a number of global gravity field models: GEM-T2, JGM2, 3, EGM96, TEG4, GRIM5-S1, -C1, POEM GS 01, PGM 2000A, EIGEN-1S, 2 (e.g., Wagner and Klokočník 1994; Klokočník and Wagner 1994; Wagner et al. 1995a,b; Wagner et al. 1997b; Klokočník et al 1996, 1998, 1999, 2002, 2004) with an extension to dual-satellite crossovers, Klokočník et al (2000a).  Recently we tested EIGEN-3p (also called 03S), EIGEN-GRACE 02S, TUGRAZ 04, UTCP 03, and EIGEN-GL04C  (results not yet published).
      Following (Klokočník and Kobrle 1992, Klokočník et al., 1992), for the SSCs, we may write




[image: image34.png]LK
s W
rfw}fm""

": o ﬂﬂ”"‘” ”f "' ‘l'*i " ! ‘\‘ \::R;M{
I ::’ ’ ! ‘ \ )
) I’fff’ff J -

Sl mfr;,

; r.fl'ffff’ 6‘ ' .. “s‘
I/ R sl ,"' ,J’ ‘ ‘
H’t L)

)
'Pf;ﬂ-"r‘ H-"fff’ ’#'
AN

. ¥ e,
M ;;;.«;, f!;; 'fJ ’a t

o+,
TN gt 20
11y 0‘ SR AL RN
i ’ “\\\&\ :".'#‘."f (PR
s






            (1)

where 
[image: image2.wmf](

)

I

,

,

a

C

m

f

d

, 
[image: image3.wmf](

)

I

,

,

a

S

m

f

d

 are corrections to the LLCs for crossovers due to the full geopotential:



[image: image4.wmf]lm

lm

l

l

m

l

m

C

Q

C

2

max

=

=

å

=




[image: image5.wmf]lm

lm

l

l

m

l

m

S

Q

S

2

max

=

=

å

-

=




(2)

where I is the orbital inclination, a the semimajor axis of the satellite orbit, and the Qs are the resulting influ​ence functions (Rosborough 1986) in metric units, and 
[image: image6.wmf]f

 is geocentric latitude. The harmonic coefficients Clm, Slm  and the influence functions Qlm (I,a,
[image: image7.wmf]f

)  are fully normalized.
     Latitude lumped coefficients (LLC), based on long-term averaged SSC data, can be (and were) used to test many global gravity models (namely their lower degree and order portion),  as described in detail in Klokočník et al. (2000b, 2002). A short review of the method is available as a poster on request (Klokočník et al, 2006).
     Our method consists of the following three steps:
     STEP 1: 
We use observed SSCs after all available altimeter corrections,  then convert them to observed latitude lumped coefficient (LLC) discrepancies  according to Eq. 1 by a least squares adjust-ment (with some constraints on land areas without altimetry data).

     STEP 2:  

We compute the corresponding LLC errors for the geopotential used for the observed orbit  from its variance-covariance matrix of spherical harmonic coefficients (usually tentatively scaled or already calibrated)  


[image: image8.wmf](

)

(

)

å

å

å

å

=

=

=

=

=

=

max

1

2

1

max

2

2

1

max

1

2

1

max

2

2

1

.

,

4

,

,

4

2

2

l

m

l

m

l

m

l

l

m

l

S

m

l

S

m

l

S

l

m

l

m

l

m

l

l

m

l

S

m

l

S

m

l

C

S

S

COVAR

Q

Q

C

C

COVAR

Q

Q

m

m

s

s

                                                           (3)

     STEP 3:  

Finally, we compare results of STEP 1 and STEP 2 in a statistical way since STEP 1 yields actual discrepancies while STEP 2 only expected squared errors.  We can do this comparison of LLCs (observed/computed) for each latitude band but normally we summarize the result by geopotential order for all bands. This comparison either confirms the (tentative) scale/-calibration factor of the covariance matrix tested or leads to suggestions for its change.
     One example is in Figs. 1a-c,  namely STEP 1 (upper left), STEP 2 (upper right), and STEP 3 (below). Recent testing of the newest gravity models based solely or dominantly on the CHAMP/GRACE data  revealed a decrease of the accuracy at few lowest orders, namely at order  m = 2  (Kostelecký et al, 2006).   Fig. 2  shows more about these new results.
     Is satellite altimetry precise enough to test the newest gravity models with extensive sets of the CHAMP data? Radial orbit improvement with the CHAMP data is remarkable - from about 1 meter with GRIM5S1 to several centimeters with EIGEN 2, 3p and to subcentimeter level with the newest EIGEN 04, for the orbit of CHAMP.
     The radial error may decrease also for the other orbits with inclinations and semimajor axes different from those of CHAMP.  For the ERS orbit type, SSC error was about 4 cm with GRIM5S1 and is about 3 cm with EIGEN 1S, for Geosat ~11/8 cm respectively. Let us consider a signal to noise  (s/n)  ratio, where s means  the SSC errors  from the covariance projections and  n  means the SSC data errors. Due to the long-term averaging of SSC (sometimes several years),  we are able to keep  n  below 1 cm; for the best ERS1/2, Envisat and Geosat data available in 2003,  n is about  0.6/0.9 cm. Thus, s/n was and still is high enough to have our test statistically significant.

     Is satellite altimetry precise enough to test the newest gravity models with extensive sets of the GRACE data? Gravity field models with the GRACE data are already more precise than satellite crossover altimetry. However, common trends appear in the results of testing with the LLCs for CHAMP and GRACE based models, see Fig. 2. It means that also for the GRACE based models there is a problem of the accuracy at the lowest orders of harmonics.


[image: image9.emf]RMS of power of LLC discrep./errors for Geosat, model EIGEN 2

-1

0

1

2

3

4

5

6

0 10 20 30 40 50 60 70 80

order m

cm

EIGEN 2 crossovers (lin. transf.)

EIGEN 2 covar (70x70) formal (=descaled)

EIGEN 2 covar (70x70) scaled


Figs. 1 a,b,c.: Example of application of accuracy assessment of a gravity model (EIGEN 2) using SSC (Geosat) with LLC, for more details see Eqs. 1 – 3. (Klokočník et al., 2002).


[image: image10.emf]
Fig. 2: Measured and projected errors in LLCs from Envisat SSC with three GRACE models. First model (EIGEN-GRACE01S) is based only on the GRACE data, the second model (EIGEN-CG01C) is a combination solution from CHAMP, GRACE and surface data, and the third model (EIGEN-GL04C) uses GRACE, LAGEOS and surface data. The LLC errors projected from the covariance matrix of EIGEN-GRACE02S (a model similar to other models used) are shown in grey, first directly from the formal covariance matrix, second from that matrix scaled by factor 5. We see a significant discrepacy between the smaller projected LLC errors from covariances and the larger ones derived from „observed“ altimetric SSCs, namely for the lowest orders. The problem is still open.

Figure © Carl A.Wagner, 2006 

2.  SINGLE AND DUAL SATELLITE CROSSOVERS DATA (SSC and DSC) 

TO REFINE GRAVITY FIELD MODELS

     In our older experiments, we were interested in the role of SSC and DSC data, independent of existing gravity models, in determination of Clm, Slm. We took JGM 3 model (now very old) and we added SSC and DSC between T/P, ERS 1 and Geosat. We tried to „refine“ JGM 3 (Wagner et al, 2000). The experiment can be repeated with new CHAMP/GRACE-only models, provided that we would have excellent SSC/DSC,  corrected for all accessible environmental error, say from JASON, ERS-2 or ENVISAT. Then, we can expect detectable changes in values of some of the lower degree and order harmonic coefficients.  

     Moreover, in  (Wagner et al, 2000), Appendix B, we explain (we think for the first time) the case of statistical expectation for semi-independent solutions in LSE, taking fully independent and completely dependent solutions as special cases.


[image: image11.emf]NOAA  DSC, ERS - T/P, AD,  (Oct92 - Dec93)

NOAA  DSC, ERS - T/P, AA,  (Oct92 - Dec93)

NOAA  SSC, T/P,  (Oct92 - Jan95, Cy 2-14)

NOAA  SSC, ERS,  (Apr92 - Dec93, Cy 1-18)

NOAA  SSC, Geosat,  (Apr85 - Nov88, GM+ERM)

c)

b)

a)

NOAA  DSC, Geosat - T/P, DD, 8 yr. gap (85apr93 - 88oct96)

NOAA  DSC, Geosat - T/P, AD, 8 yr. gap (85apr93 - 88oct96)

NOAA  DSC, Geosat - T/P, AA, 8 yr. gap (85apr93 - 88oct96)

NOAA  DSC, ERS - T/P, DA,  (Oct92 - Dec93)

NOAA  DSC, ERS - T/P, DD,  (Oct92 - Dec93)

a)

b) c)

Fig. 4, part 1



Fig. 3. Illustration of inversion using SSC and DSC from   ERS 1, T/P, and Geosat  (Wagner et al, 2000) to try to refine older JGM 3. Columns (a) contain the data, i.e. SSC or DSC, columns (b) show their precision, and columns (c) depict residuals after the inversion. Reader can found in (Wagner et al, 2000) the series of color figures for all SSC and combinations of DSC used in that analysis.

3.  USE OF DSC BETWEEN LOW AND HIGH ORBITS TO DETECT OFFSET
AMONG TERRESTRIAL FRAMES BETWEEN ALTIMETRY MISSIONS.
     There is one type (AD) of  the single-satellite crossovers (SSC) between the ascending (A) and descending (D) track of the given orbit, just four types (AD, DA, AA, and  DD) of the dual-satellite crossovers (DSC) between the relevant tracks of a pair of satellite orbits, and just twelve their combinations (AA+AD, AA-AD, AA+DA, AA-DA, DD+DA, DD-DA, DD+AD, DD-AD, AD+DA, AD-DA, DD+AA, and DD-AA), taking the DSC in pairs (Klokočník and Wagner, 1999). Each quantity has different sensitivity to the gravity field parameters and in turn, also different gravity induced error (e.g., Klokočník et al, 1993, 1995, 1999, Wagner et al, 1997b).  In the first use of DSC, Shum et al (1990) intended to surrogate SSC of the lower orbit by DSC between lower and higher satellite (but they used only the AD type).  More generally, there are very interesting properties of some of the combinations mentioned above, which might be used as a diagnostic tool in the study of residual error effects in environmental corrections of altimetry data (references below).

     We used DSC of the lowest degrees to test possible offset between coordinate systems of the higher, more precise altimetry mission with better determined orbit (like TOPEX-/Poseidon) and a lower flying, less accurate mission (like Geosat) [e.g., Bosch et al, 1998]. This task can be repeated for any pair of altimetry missions (like Jason and Envisat or GFO). The DSCs and the offset detection and removal is useful to interconnect series of SSCs from different (desparate) missions even with time gaps  (like T/P-Geosat). 
     Here we recall our results from Wagner et al (1997a,b)  and we show one of examples of  our analysis (1997a)  on  Figs. 4a-d.

For the DSC, type AA we have


[image: image12.wmf](

)

(

)

l

l

cos

sin

11

11

2

,

1

11

11

2

,

1

11

11

2

,

1

11

11

2

,

1

1

,

1

:

12

S

dQ

C

dQ

C

dQ

S

dQ

X

S

C

S

C

m

l

AA

+

+

+

=

D

=

=

                               (4)

which can be transformed to the well known formula for the offset between two coordinate systems:


[image: image13.wmf][

]

[

]

s

correction

cpr

dz

dy

dx

X

h

m

rel

1

sin

sin

cos

cos

cos

1

,

0

)

.(

12

+

+

+

=

D

=

D

-

=

f

l

f

l

f

                  (5)

One example of our analysis with TOPEX and Geosat is shown in Figs. 4a-d and the following values for the offset were found   dx =  +1.0±0.1 ,  dy = -7.0 ±0.2 . 

     DSC residuals and their combinations are based on observations, thus they consist of various gravitational and non-gravitational effects. The non-gravitational parts of the DSC comes from the offset, but also from the remaining errors after the altimetry corrections are applied, including empirical one like 1 cpr, and from residual errors of tidal models, too. Unknown amount of the DSC signal suffers from an aliasing between the non-gravitational and gravitational signal.  Finally, there is also white data noise.

     As an analogy of gravity-induced geographically correlated (not depending on sense of track and time) and  variable (anticorrelated) portions of radial component or error, we may introduce non-gravitational correlated and variable components. We will call non- gravitational correlated error such an error of non-gravitational origin which does not depend on sense of track and is (quasi)-constant over some long-term interval. The variable (anticorrelated) error of non-gravitational origin will be such an error which varies with track and time. Fast variables within orbit arc or during the period used to compute the crossovers should be averaged out, slower variables will have some effect on the crossover residuum.

[image: image1.wmf](

)

(

)

(

)

[

]

l

f

d

l

f

d

l

f

m

a

S

m

a

C

X

m

m

m

m

m

cos

,

,

sin

,

,

,

max

1

I

+

I

å

=

D

=

=

     The coordinate frame offset between two missions is typical example of geographically correlated non-gravitational phenomenon. It does not depend on type of the DSCs (i.e. on sense of satellite's track, and on time). Also mean sea surface would belong to this category. Media corrections are typical variable components as they differ for the crossovers between the tracks. The long-term 'month-to-month' averaging is effective mostly for seasonal effects. Long-term effects (like a quasi-permanent part of the ocean currents, and partly ENSO effects) cannot be averaged by this way and a part of them remains, in our terminology, as geographically correlated non-gravitational signal.

Figs. 4a-d.   The example of applications of DSC from two altimetry missions (Geosat and TOPEX/Poseidon) to find relative coordinate frame offset of the lower and less accurate with respect to the higher and more precise (taking as errorless) mission (here Geosat vs T/P, model JGM3, NOAA data. a) AA duals before removing geocenter shift, b) AA duals after removing geocenter shift, c) DD duals before removing geocenter shift, d) DD duals after removing geocenter shift. More in (Wagner et al. 1997a,b; Kostelecký and Klokočník 1999).

     More about combinations of DSC and their applications is in Klokočník et al (1999, 2000a),  Klokočník and Wagner (1999), and Wagner et al (1997b). They are found useful for error analysis in conjuction with applications of DSC in geodesy and oceanography. One kind of combination, when applied to the dual missions in a high-low orbit configuration, approximates the dominant „mean“ part of the total radial perturbations of the lower orbit. Another kind yields no geopotential error and can help elucidate oceanographic changes between passes, among other error of altimetry corrections. A third kind is a surrogate of SSC data and can be used to check the significance of the background (non-geopotential) errors that are not common to the two kinds of crossovers. The levitating combinations (with zero gravity effect), namely  (DD-DA) and (AD-AA) of ERS1 & T/P and their comparison with SSC of T/P are shown on Figs. 5a-c, taken from Klokočník et al (2000a),  sect 3.5. 

[image: image14.png]90" 180" 270"
PATHFINDER (AD-AA) DSC Diff., tms=2.9cm

90° 180" 270 o 90 180 270
Tndependent NOAA data (AD) T/P SSC, tms =2.9cm Tndependent Pathfinder data (AD) T/P SSC, tms = 1.9 cm

DSC ERS1-T/P levitating combinarions (=T/P SSC "Surrogares”) versus T/P SSC dara

5 4 3 2 1 0 1 2 3 4 5 6 7 8





Figs. 5a-d. The „ levitating“ combinations (AD-AA) of ERS 1 & T/P, where the crossovers for ERS 1 are based on DGM E04 gravity model (Delft) and the crossovers for T/P on JGM 3 (NOAA). One source of data is NOAA (Fig. 5a) upper left, another is from Pathfinder NASA (Fig. 5b), upper right.  The combinations (Figs 5a and b) provide a T/P ‚surrogate‘.  They compare to the T/P SSC ‚originals‘ (Figs. 5c and d;  bottom). These figures support the hypothesis that the NOAA data of T/P suffered from an inconsistency, probably in media corrections, recommended to NOAA for T/P prior  to 1997 (in contrast with those used later).

4.  GEOPOTENTIAL AND DIFFERENCES AMONG HEIGHT SYSTEMS
     Here we note on use of satellite altimetry for determination of gravity potential of the geoid and definition of the world height systems. We start with formula for the gravity potential in spherical coordinates

     
[image: image15.wmf](

)

(

)

(

)

(

)

lm

lm

lm

lm

lm

l

l

m

l

l

e

S

C

GM

r

f

r

P

m

S

m

C

r

a

r

GM

r

W

,

,

,

;

,

,

cos

2

1

sin

sin

cos

,

,

2

2

2

2

0

w

l

j

j

w

j

l

l

l

j

=

=

+

+

=

å

å

¥

=

=

           (6)                                                                                                                                               

where 
[image: image16.wmf]l

j

,

,

r

 are spherical coordinates of the point, GM geocentric gravity constant, Clm, Slm fully normalized Stokes’ parameters (harmonic coefficients) of the Earth’s gravity field, 
[image: image17.wmf](

)

j

sin

lm

P

 are fully normalized Lagrange associated functions and ω is rotational velocity of the Earth.

     For the potential we can write

     
[image: image18.wmf](

)

(

)

lm

lm

geoid

geoid

geoid

S

C

GM

r

f

r

W

,

,

,

;

,

,

,

,

w

l

j

l

j

=

,                                                         (7)

and ‘altimetry equation’ yields
    
[image: image19.wmf](

)

(

)

(

)

2

1

,

,

,

D

-

-

D

-

=

l

j

l

j

l

j

h

r

r

s

geoid

                                                                         (8)

where 
[image: image20.wmf](

)

l

j

,

s

r

 is the geocentric height of altimetric satellite determined from precise orbit determination,  Δ1 includes calibration, tidal and SSH corrections, 
[image: image21.wmf](

)

l

j

,

h

 is altimetric measu-rement and  Δ2 is  a „geometric correction“ (altimetric measurement is directed to plumb line, we use spherical coordinates).

[image: image22.wmf]
                                      Table 1 – taken from Burša et al., 1999

     Theoretically, one point with altimetry data is enough to obtain a solution  for Wgeoid, but in practice we have available many sites for the solutions. The values of Wgeoid, based on Eqs. (7) and (8) and the reference gravity field model EGM96, were determined as a mean value of individual point solutions for particular oceans and then for the whole Earth. If  Eqs. (7) and (8) are used separately for different oceans, then we can determine differences between the mean gravity potential Wgeoid = W0  and Wocean,  and the mean height differences Δh between individual oceans - see Table 1, taken from (Burša et al., 1999).  The T/P data in time interval 1993 – 1996 was used. The mean value determined by (Burša et al., 1998) is:
Wgeoid = (62 636 855.611 ± 0.5) m2 s-2.

“Altimetric” gravity potential of the geoid Wgeoid is a fundament for determination of the “common world height system” if we use additional results from “GPS/levelling”. Principle of this method is obvious  from  Figure 6.

     For a terrestrial point  P the geopotential WP is (Burša et al. 2001)

      
[image: image23.wmf](

)

(

)

lm

lm

P

P

P

P

P

P

P

S

C

GM

h

f

h

W

,

,

,

;

,

,

,

,

w

l

j

l

j

=

                                                         (9)

where ellipsoidal coordinates 
[image: image24.wmf]P

P

P

h

l

j

,

,

 are determined by GPS positioning. The value of Worigin is computed by gravimetry and by levelling from WP. 

      For the „height anomaly“ ΔH of individual height system the following equation is valid

    
[image: image25.wmf]g

origin

geoid

W

W

H

-

=

D

                                                                                                (10)

where  γ is mean value of gravity. Results from Burša et al. (2001, 2004) are here presented in  Tables 2 and 3. The differences between the five basic world height systems are in Table 2 and have accuracy better than  0.1 m.

[image: image26.png]Wp - Worigin determined
from gravimetry
and levelling

hp (levelling)

origin of height
system

Worigin
Wgeoid

corrections c

hp
Amani determined determined P(geoid)
geoid from altimetry from GPS

reference ellipsoid





Fig.6.  Value of  ΔH  is estimated by two ways (dynamical and geometrical), as was described by Eqs (6-8).
    If we know „sea level height“ hP(levelling) of the point P, geometric access to solve this problem is possible. From Figure 6 we have

    
[image: image27.wmf])

(

)

(

geoid

P

levelling

P

P

h

h

h

H

-

-

=

D

                                                                                  (11)

where hP(geoid)  is the ellipsoidal height of geoid in P.  It is determined by inversion of Eq. (6) for rgeoid.  If detailed gravimetric data is at disposal, then the gravity potential, Eq. (6), can be completed by additional potential T. 

[image: image31.png]Vertical position error of reflecting points between GPS and CHAMP,
std of measurement = 20 mm

200
----- d: CH=100/200, GPS=50/100
160 . S *
\‘\ —— sd: CH=50/200, GPS=50/100
—cd: CH=50/100, GPS=50/100
.g. 120 . O o sd: CH=20/50, GPS=50/100
£
| .
o
=
® 80 S
40
0 T T T T T T
20 30 40 50 60 70 80

gamma [deg]

90




[image: image32.png]400

350

300

250

200

error [mm]

150

100

50

Error of measurements versus orbital errors [mm]

------ sd meas 20

— — —sd meas 40

sd meas 60

=— = sd meas 80
— =sd meas 100

sd meas 200

sd:CH=50/100, GPS=50/100

gamma [deg]




[image: image28.wmf]
AHD – Australian Vertical Datum, NAP – Normal Amsterdamsch Peil, KHD – Kronstadt Height Datum, NAVD88 – North American Vertical Datum 1988, N60 – Finnish Height Datum

Table 2 – differences of geopotential at several tide gauge stations and corresponding vertical shifts ΔH (reproduced from Burša et al., 2001).

[image: image29.wmf]
Table 3   –   geopotential values at local vertical datums LVD.  The quantity  δH0i  is the vertical shift of the LVD origin, related to the reference surface W-W0  (Burša et al., 2004).

5.  THEORETICAL INVESTIGATION  OF  BISTATIC SATELLITE ALTIMETRY
5.1. Principle of  bistatic satellite altimetry.

     Instead of one satellite, two satellites cooperate and sender S2 sends signal to receiver S1 directly (d12)  and indirectly, the indirect signal being reflected off the ocean surface (d1 + d2), see Fig. 7. The observed quantity in bistatic altimetry (BA) is the time or distance difference between those two (s/c "bistatic radar value").

     The method was suggested by (Martin-Neira, 1993). It was tested and is used on the ground at lakes or from aircrafts (e.g., Treuhaft et al., 2001; Lowe et al, 2002). The most precise GPS lake altimetry measurements from a fixed site receiver retrieved lake surface height with 2 cm precision (Zuffada et al., 2002, 2004). We have to mention the short space technology experiment on the SLR-2 Space Shuttle mission (in 1994); it recorded about 15 hours of BA data from GPS, gathered in the high resolution mode. Technology experiments on CHAMP and SAC-C were also performed, but scientifically useful space data is not yet available. A dedicated bistatic altimetry mission is highly required (e.g. Komjathy et al, 1999; Le Traon et al, 2002, 2003;  Ruffini et al 1999, 2000; Wiehl et al 2003; Zuffada et al 2002, 2004).

     We studied geometry and accuracy of the BA concept assuming specular reflecting points on the ocean/sea surface (Wagner and Klokočník, 2003; Kostelecký et al, 2005).

[image: image30.wmf]
Figure 7:    The concept of satellite bistatic altimetry.

5.2. Geometry

     In conventional satellite altimetry all orbits have been in repeat tracks with cycle periods less than about 1 month since Seasat  except for the geodetic missions (GM). But these GM were specifically designed to generate detailed mean sea surfaces, not to monitor ocean dynamics (e.g., Lemoine et al, 1998). Long term simulations show that complementing a traditional satellite-altimeter (short cycle) ERM with the capability of bistatic altimetry will yield a dense pattern of returns (spatially and temporally) that could be used for either geodetic or oceanographic purposes.

      For example, we computed number and distribution of reflecting points for 24 GPS satellite and CHAMP or SAC-C for 180 day intervals of BA observations with given reasonable rate of observations. We found several times higher number of BA points than would belong to traditional nadir radar altimetry.  We have also one hypothetical example for 24 GPS-T/P configuration, although T/P is not equipped for BA (Kostelecký et al, 2005). We wanted to show the geographic distribution of the reflecting points; noting the distribution of reflection tracks in specific 1 degree bin in the North Atlantic which (being close to the northern edge of the Gulf Stream) may show strong eddy and ring activity.  This area is not covered at all in the conventional T/P mission.  With reflection tracks from all 24 GPS satellites, downward-looking receivers in a hypothetical "T/P" orbit would pick up a remarkably dense amount of sea height information. The key to the power of bistatic altimetry is the dispersion of the returns away from the lower-satellite track which fills in the limited coverage of the conventional mission. This dispersion is a function of both the altitude of the receiver orbit  and the minimum reflection horizon angle permitted. 

5.3.  Accuracy

     For the accuracy estimates, we consider not only measurement errors (in d12) but also orbit errors (in the state vectors of S1, S2) and we ask how large error can be expected in the vertical (radial) position of  a theoretical reflecting point P in a ‚model situation‘ (on spherical Earth).

     Numerical results (Kostelecký et al, 2005), here in Figs. 8a,b, were computed with this data:

1. GPS orbit error is  min/max = 50/200 mm radially; 100/500 mm cross-radial (Mader, priv. commun. 2002; materials of Interntl. GPS Service, 2000).

2. CHAMP orbit error is min/max = 20/100 mm, radially; 50/200 mm cross-radial  (Reigber, priv. commun. 2002).

3. Delay measurement errors min/max = 20/200 mm (minimum estimate based on GPS-fixed ground antennae experiment at 500 m over Crater Lake, Truehaft et al., 2001; this figure may be optimistic for a receiving spacecraft in motion at 400 km altitude [CHAMP]).  The range from  200 to 20 mm expresses our wishes for the future accuracy level.

     Obviously the accuracy degrades slowly with the off-nadir angle to about 50 degrees, then rapidly (Fig. 8a). Thus, we can expect many BA points off-nadir with promising accuracy, comparable to that of the traditional nadir altimetry. The orbit and the measurement errors compete in the total error budget of the reflecting point position as is shown in Fig. 8b.

[image: image33.png]



Fig.  8 a,b.  Accuracy estimates of  BA  concept accounting for the measurement and orbit errors. The angle „gamma“ is defined in Figure 7.   (8a) on left: various orbit errors of CHAMP and a constant orbit error of GPS satellites and measurement error, (8b) or right: various measurement errors for fixed orbit errors.

5.4. Position of reflecting points on ocean


     The spherical approximation of the reflecting surface (the ocean) is sufficient for geometry and accuracy treatment, but in reality we need to compute position of the reflecting point on geoid. Olivík et al (2006)  have  developed three methods that are able to do it under different assumptions (one example in Fig. 9).

Fig.  9  -  Illustration to one method of solving position of reflecting point on rotational ellipsoid representing the Earth (Olivík et al, 2006), using three quadrics: the quadric Q1 with foci in S1 and S2,  the straight circular cone Q2 with vertex in S1, and the quadric Q0 as a reference geocentric ellipsoid of the Earth.

Acknowledgements. Close, intensive and extremely fruitful cooperation with Carl A. Wagner and others from NOAA, with R. Scharroo (Delft at that time), and Ch. Reigber, P. Schwintzer and Ch. Foerste (GFZ) is highly acknowledged.  This investigation has partly been done in frame of the grant # A30074 from GA AV of the Czech Republic  and supported by the project LC 506 (CEDR) by  Ministry of Education, Youth and Sports of the  Czech Republic.
REFERENCES
Bosch W., Klokočník J., Wagner C.A., Kostelecký J., 1998, Geosat and ERS-1 Datum Offset Relative to TOPEX/Poseidon and Geopotential Corrections Estimated Simultaneously from Dual-Satellite Crossover Altimetry,  pres. at symp. IAG Sect II: „Towards an Integrated Global Geodet. Obs. System“,  5-9 Oct. Munich 

Burša, M., Kenyon, S., Kouba, J., Šíma, Z., Vatrt, V.,Vojtíšková, M., 2004, A global vertical reference frame based on four regional vertical datums. [Globální výškový systém založený na čtyřech regionálních výškových systémech.] Studia geophysica et geodaetica. Roč. 48, č. 3, s. 493-502

Burša, M., Kouba, J., Muller, A., Raděj, K., True, S. A., Vatrt, V., Vojtíšková, M., 2001, Determination of geopotential differences between local vertical datums and realization of a world height system. Studia geophysica et geodaetica, 45, No 2, p. 127-132

Burša, M. Kouba, J., Müller, A., Raděj, K., True, S. A., Vatrt, V., Vojtíšková, M., 1999, Differences between mean sea levels for the Pacific Atlantic and Indian Oceans from TOPEX/POSEIDON altimetry. Studia geophysica et geodaetica, 43, No 1, p. 1-6

Burša, M., Kouba, J., Raděj, K., True, S. A., Vatrt, V., Vojtíšková, M., 1998, Mean Earth's equipotential surface from TOPEX/POSEIDON altimetry.  Studia geophysica et geodaetic,  42, No 4 , p. 459-466 

Colombo O.,1984, Altimetry, Orbits and Tides,  NASA Techn.  Memo., TM 86180

Klokočník, J., Kobrle, F., 1992, Dual-Satellite Crossover Altimetry for ERS-1/TOPEX,  pres. at  COSPAR "The World Space Congress", Washington,  DC, paper B.9-M.2.05, see also: Adv. Space Res.  13, (11) 335-337.

Klokočník J., Wagner, C. A., Kobrle, F., 1992, A Test of GEM~T2 from GEOSAT Crossovers using Latitude  Lumped Coefficients, presented at VIIth Interntl. Symp.: "Geodesy and Physics of the Earth", Potsdam Germany,  Proceedings IAG Symp. 112, eds. H.Montag and Ch. Reigber, Springer Verlag, pp. 79-82.

Klokočník, J., Kostelecký, J., Jandová, M., 1993, Altimetry with Dual-Satellite Crossovers,  presented at IAG GM Beijing, China, Session D~10: "Impact of Satellite Altimetry on Gravity Field Determination",  Manuscr. Geod.,  20, 82.

Klokočník, J., Wagner, C. A., 1994, A Test of GEM T2 from GEOSAT Crossovers using Latitude Lumped Coefficients,  Bull. Geod.,  68, 100.

Klokočník, J., Wagner. C. A., Kostelecký. J., Jandová.  M., 1995, Altimetry with Dual-Satellite Crossovers,  Manuscr. Geod.,  20,  82 - 95.

Klokočník, J., Wagner, C. A., Kostelecký, J., 1996, Accuracy Assessment of Recent Earth Gravity Models Using Crossover Altimetry,  Studia geoph. et geod.,  40, 77-110.

Klokočník, J, Lemoine, F. G., Kostelecký, J.,  1998, Reduction of Crossover Errors in the Earth Gravity Model (EGM 96),   Marine Geod., 21, 219-239.

Klokočník, J., Wagner, C. A., 1999, Combinations of Satellite Crossovers to Study Orbit and Residual Errors in Altimetry,   Celest. Mech. and Dynam. Astr.  74, 231-242.

Klokočník, J., Wagner, C. A., Kostelecký, J., 1999, Spectral Accuracy of JGM~3 from Satellite Crossover   Altimetry,   J. Geod.,  73, 138--146.

Klokočník, J., Wagner, C. A., Kostelecký, J., 2000a, Residual Errors in Altimetry Data Detected by Combinations  of  Single- and Dual-Satellite Crossovers,  J. Geod.,  73, 671--683.

Klokočník, J., Reigber, Ch., Schwintzer, P., Wagner, C. A. , Kostelecký, J., 2000b,  Evaluation of pre-CHAMP Gravity Models GRIM5-S1 and GRIM5-C1 with Satellite Crossover Altimetry,  Sci. Techn. Rep. STR 00/22, GFZ Potsdam.

Klokočník, J., Reigber, Ch., Schwintzer, P., Wagner, C. A., Kostelecký, J., 2002, Evaluation of pre-CHAMP Gravity Models GRIM5-S1 and     GRIM5-C1 with Satellite Crossover Altimetry,  J. Geod. 76, 189-198.

Klokočník, J., Kostelecký, J., Novák, P., 2004, On future of gravity field models accuracy assessment, poster at EGU, Nice.

Klokočník, J., Kostelecký, J., Wagner C.A., Gruber Ch., 2006, Review of the use of satellite crossover altimetry to test the accuracy of the Earth gravity models, poster at  symp. „15 years of progress in radar altimetry“, Venice 13-18 March.

Kostelecký, J., Klokočník, J., 1999, Detection of coordinate frame offset from satellite altimetry,  EUREF Publ. #7/1, Verlag des BKG, Frankfurt am Main, pp. 230-232.
Kostelecký, J., Klokočník, J., Wagner, C. A., 2005, Geometry and accuracy of reflecting points in bistatic satellite altimetry, J. Geod.  79, 421-430.

Kostelecký, J., Klokočník, J., Wagner, C. A., Scharroo, R., Gruber, Ch., Bezděk A. Doornbos, E., Novák, P., 2006, Degradation in accuracy of CHAMP/GRACE only Earth Gravity Field Models, EGU Vienna 2006.

Komjathy, A.,  Garrison, J. L.,  Zavorotny V. , 1999,    GPS: A new tool for Ocean science,  GPS World, April, 50-56.

Lemoine, F.J., Kenyon, S.C., Factor, J.K., Trimmer, R. G., Pavlis, N.K., Chinn, D.S., Cox, C.M., Klosko, S. M, Luthcke, S.B., Torrence, M.H., Wang, Z.M., Williamson, R.G., Pavlis, E. C., Rapp, R. H., Olson, T.R., 1998,  The Development of the Joint NASA GSFC and the National Imagery and Mapping Agency (NIMA) Geopotential Model EGM 96,   NASA/TP-1998-206861.

Lerch  F.J.,  1991,   Optimum  data  weighting  and  error calibration  for estimation  of gravitational parameters, 

Bull. Geod. 65, 44-52.

Le Traon, P.Y., Dibarboure, G., Ruffini, G., Cardellach E. , 2002, Paris Beta, Study of requirements and mission definition for bistatic altimetry, Starlab et al, ESA/ESTEC Contract Rep. 15083/01/NL/MM, Techn. note extract from the Paris-Beta ESTEC/ESA study.

Le Traon, P.Y., Dibarboure, G., Ruffini, G., Germain, O., Thompson, A., Mathew, C., 2003,  Paris Gamma,    GNSS-R measurement for ocean mesoscale circulation mapping - an update, Starlab, ESA/ESTEC Contract Rep. TRP ETP 137A.  Workshop on oceanography with GNSS Reflections, Barcelona.

Lowe, S.T. et al., 2002, 5-cm precision aircraft ocean altimetry using GPS reflections, Geophys. Res. Letts. 29:10.

Martin-Neira, M., 1993, A passive reflectometry system: application to ocean altimetry,  ESA Journal 17: 331-356.

Novák, P., Kostelecký, J., Klokočník, J., 2006, Testing new geopotential models through comparison of high-resolution quasi-geoid models with GPS/elevation data,  poster at EGU Vienna

Olivík, S., Kočandrlová, M., Kostelecký, J., Klokočník, J., 2006, Position of reflecting points in bistatic satellite altimetry: theoretical solutions for ellipsoid,   J. Geod.,  in review,  also poster  EGU meeting, Vienna.

Rosborough, G. W., 1986, Satellite Orbit Perturbations due to the Geopotential, report CSR-86-1  Univ. of  Texas at  Austin, Center for Space Research

Rosborough, G. W., Tapley, B. D., 1987,  Radial, Transverse and Normal Satellite Position Perturbations due to  Geopotential,  Celest. Mech.  40, 409-421

Ruffini, G., Cardellach, E., Rius, A., Aparicio, J.M., 1999, Remote sensing of the ocean by bistatic radar observations: a review, IEEC Rep. WP 1000, ESD-iom 019/99, Earth Sci. Dept, Radar Group, Barcelona, 94 pp.

Ruffini, G., Soulat, F., 2000, PARIS Interferometric Processor analysis and experimental results, theoretical feasibility analysis, IEEC-CSIC Res. Unit., Barcelona, PIAER-IEEC-TN-1100/2200, ESTEC Contr. No. 14071/99/NL/MM,    \\ ftp:/ftp.estec.esa.nl/pub/eopp/pub/

Shum, C. K., Zhang, B. H., Schutz B. E., Tapley, B. D.,  1990,  Altimeter Crossover Methods for Precison Orbit Determination,  J. Astronaut. Sci.,  38,  355

Truehaft, R., Lowe, S., C. Zuffada, C., Chao, Y. , 2001,  2-cm GPS-altimetry over Crater Lake,  Geophys. Res. Letters 28:23, 4343-4346.

Wagner, C.A.,  1989, Summer School Lectures on Satellite Altimetry,  Lect. Notes Earth Sci.  25,  Theory of Satellite Geodesy  and Gravity Field Determination, eds.  F. Sanso and  R. Rummel, pp. 285--334, Springer-Verlag, New York

Wagner, C.A., , Klokočník J., 1994,  Accuracy of   the GEM T2 Geopotential from GEOSAT and ERS-1 Crossover  Altimetry,  J. Geophys. Res.  99, B5, 9179--9201

Wagner, C.A., , Klokočník J, Kostelecký J, 1995a,  Altimetry with Dual-Satellite Crossovers,  Manuscr. Geod.,  20,  82--95

Wagner, C.A., Klokočník J, Tai C.K, 1995b, Evaluation of JGM 2 Geopotential Errors from Geosat,    TOPEX/Poseidon and ERS-1 Crossover Altimetry, presented at the XXXth COSPAR Plen. Meet., "Panel on Satel. Dyn.",  Hamburg, 1994; see also: Adv. Space Res.,  16, (12) 131--141

Wagner, C. A., Klokočník J, Cheney R. E., 1997a, Making the Connection between Geosat and  TOPEX/Poseidon,   J. Geod., 71,  273-281

Wagner, C.A., Klokočník, J., Kostelecký, J., 1997b, Dual-Satellite Crossover Latitude Lumped Coefficients, their use in Geodesy and Oceanography,   J. Geod., 71, 603--616

Wagner, C.A., Klokočník, J., Kostelecký, J., 2000, Geopotential and Oceanographic Signals from Inversion of  Single and Dual satellite Altimetry,   VUGTK  Techn. Rep. 46, 26, Zdiby.

Wagner, C.A., Klokočník, J., 2003,  The value of ocean reflections of GPS signals to enhance satellite altimetry: data distribution and error analysis,  J. Geod.,  77: 128-138

Wiehl, M., Legresy, B., Dietrich, R., 2003, Potential of reflected GNSS signals for ice sheet remote sensing,    Progress in Electromagnetics Research (PIER)}, 40:  177-205

Zuffada, C., Elfouhaily, T., Lowe, S., 2004,  Sensitivity analysis of wind vector measurements from ocean reflected GPS signals, Remote Sensing of the Environment, 88,  341-350

Zuffada, C., Lowe, S., Chao, Y., Treuhaft R., 2002, Oceanograpgy with GPS, presented at Interntl. „Workshop on Satellite Altimetry for Geodesy, Geophysics and Oceanography“: Summer Lecture Series and Sci. Applics., Sept., Wuhan, China; Springer IAG Symp. Vol.126, pp. 193-203. Springer-Verlag Berlin Heidelberg (2004).

_75842556.unknown

_1217154165.unknown

_1217158988.unknown

_1217233214.unknown

_1217233257.unknown

_1217159092.unknown

_1217222747.unknown

_1217156798.unknown

_1217157273.unknown

_1217154189.unknown

_76287208.unknown

_1217152760.unknown

_1217153325.unknown

_1217152913.unknown

_1217150829.unknown

_76285928.unknown

_75479584.unknown

_75506020.unknown

_75507940.unknown

_75480544.unknown

_75478624.unknown

